Material Safety Data Sheet

Product Name:  Epoxy Primer					HMIS Codes:  H    F    R    P

Product Code:  PE 1731 S							 3*    3     0    J

# Section I  - Manufacturer Information

Manufacturer’s Name:  Air-Tech Coatings

Address:  1 Paradise Park Rd.  Jacksonville, AR  72076

Emergency Phone (Chemtrec): (800)424-9300     Information Phone:  (501) 985-1484

Date Revised:  10/10/10

## Section II – Hazardous Ingredients/SARA III Information

Hazardous				Occupational Exposure Limits			Vapor Pressure

Components	      	 CAS #	 	 OSHA  PEL	ACGIH  TLV   Other		Hg @ TEMP  	Weight%

*TOLUENE	     	 108-88-3		200    PPM	100    PPM		22.7      68°F	19

TITANIUM DIOXIDE    13463-67-7	5 MG/M3		10 MG/M3         5 MG/M3	        N/A	                  <5.0

AMORPHUS SILICA      7631-86-9	6 MG/M3		10 MG/M3         3 MG/M3	        N/A		5

CRYSTALLINE 	 14808-60-7	0.1 MG/M3	0.1 MG/M3		        N/A		0.15

SILICA (QUARTZ)

*STRONTIUM 	  7789-06-2	0.1 MG/M3	0.0005 MG/M3		        N/A		19.99

CHROMATE

*METHYL  	      	  108-10-1				75 PPM	          TWA  50	0.8         50°F	21

ISOBUTYL KETONE

*Indicates toxic chemical(s) subject to the reporting requirements of section 313 of Title III and of 40 CFR 372.  US Toxic Substance Control Act (TSCA):  The component chemicals in this product are listed on the TSCA inventory.  CONTAINS CHROMIUM COMPOUND AND CRYSTALLINE SILICA – SEE SECTION VI.

Section III – Physical/Chemical Characteristics

Boiling Range:  230-236 ° F				Specific Gravity  (H20 = 1):  1.3

Vapor Density:  Heavier than Air			Evaporation Rate: Slower than ether

Coating VOC:  4.24 lb/gal  (508 G/L)			Material VOC:  4.24 lb/gal  (508G/L)

Solubility in Water:  Negligible			Appearance/Odor: Yellow liquid/Solvent

Section IV – Fire and Explosion Hazard Data

Flash Point:  45 ° F					Method Used:  T.C.C

Flammable Limits in Air by Volume – Lower:  1.2%	Upper:  8.0%

Extinguishing Media:  Foam, CO2, Dry Chemical

Special Firefighting Procedures:

WEAR SELF-CONTAINED BREATHING APPARATUS WITH A FULL FACEPIECE OPERATED IN PRESSURE-DEMAND WHEN FIGHTING FIRES.  USE WATER FOG NOZZLES TO COOL CLOSED CONTAINER.

PE 1731 S    _ 		Material Safety Data Sheet		page 2 of 3

Unusual Fire and Explosion Hazards:

DO NOT USE WATER.  FLAMMABLE LIQUID, CAN RELEASE VAPORS THAT FORM FLAMMABLE MIXTURE AT TEMPERATURE AT OR ABOVE THE FLASHPOINT.  ELIMINATE AND ISOLATE FROM ALL SOURCES OF IGNITION.  COMBUSTION WILL PRODUCE CO2, CO, AND TOXIC GASSES.

Section V – Reactivity Data

Stability:	Stable

Conditions to Avoid: 	Excessive Heat

Incompatibility:	Strong Oxidizing Agents

Hazardous Decomposition or Byproducts:  	Burning can produce CO2 and/or CO

Hazardous Polymerization:  	Will not occur

Section VI – Health Hazard Data

Inhalation Health Risks and Symptoms of Exposure:

Irritation to the respiratory Tract (Nose, Throat, Lungs) and/or Narcosis

Skin and Eye Contact Health Risks and Symptoms of Exposure:

Irritating, May injure eye tissue if not properly removed.

Skin Absorption Health Risks and Symptoms of Exposure:

Slight Irritation due to solvents.

Ingestion Health Risks and Symptoms of Exposure:

Moderately toxic.  May cause nausea, vomiting, and diarrhea.  Do not take internally.  Consult Physician.

Health Hazards  (Acute and Chronic):

Crystalline silica has been classified as probably carcinogenic for humans (2A) by IARC.  Crystalline silica is also a known cause of silicosis, a non-cancerous lung disease caused by excessive exposure to crystalline silica.

Carcinogenity:	NTP?  yes	IARC Monographs?  yes 	OSHA Regulated? Yes

DERMAL AND MUCOUS MEMBRANE IRRITATION, DAMAGE WILL RESULT FROM PROLONGED EXPOSURES.  THIS MATERIAL IS LISTED BY IARC TO HAVE SUFFICIENT EVIDENCE TO BE A A CARCINOGEN FOR HUMANS (LUNGS & RESPIRATORY TRACT) BECAUSE OF THE CHROME CONTENT.

Medical Conditions Generally Aggravated By Exposure:

Respiratory problems can be increased by inhalation of vapors.

Emergency and First Aid Procedures:

Eyes – Flush with water for 15 minutes.  Get immediate medical attention.

Skin – Remove contaminated clothing.  Wash affected area with soap and water.

Inhalation – Move to fresh air.  Obtain medical attention if needed.

Ingestion – Do not induce vomiting.  Get immediate medical attention.

For Industrial Use.  Keep Away From Children.

PE 1731 S			Material Safety and Data Sheet 	       Page 3 of 3

Section VII – Precautions for Safe Handling and Use

Steps To Be Taken in Case Material is Released or Spilled:

Wear respirator, absorb with vermiculite or other absorbent material.  Eliminate all ignition sources.  Stop spill at source and prevent spreading.

Waste Disposal Method:

Destroy by Liquid Incineration.  Contaminated absorbent may be deposited in a landfill in accordance with local, state and federal regulations.

Precautions to by Taken in Handling and Storing:

Store in a dry area.  Keep containers tightly closed.  Protect from physical damage.  Emptied containers retain product residues (vapor, liquid, and/or solid).  All hazard precautions given in the data sheet must be observed.  Store below 120 ° F.

Other Precautions:

Store large quantities in buildings designed to comply with OSHA 1910.106.  Keep closure tight and container upright to prevent leakage.  Do not store near heat, sparks or flame.  Do not get in eyes.  Keep away from children.  Avoid skin contact.  Do not take internally.  For industrial use.

Section VIII – Control Measures

Respiratory Protection:

If TLV of the product or any component is exceeded, a NIOSH/MSHA jointly approved air supply respirator is advised in absence of proper environmental control.  OSHA regulations also permit other NIOSH/MSHA respirators under specified conditions.  (See you safety equipment supplier.)  Engineering or administrative controls should be implemented.

Ventilation:

Provide sufficient mechanical (general and/or local exhaust) ventilation to maintain exposure below TLV(s).

Protective Gloves:

Wear solvent resistant gloves (see you equipment supplier or safety engineer).

Eye Protection:

Chemical splash goggles suggested, other safety glasses permitted.

Other Protective Clothing or Equipment:

To prevent repeated or prolonged skin contact, wear solvent impervious clothing and boots.

Work / Hygienic Practices:

Wash with soap and water after use, before eating or smoking.

Section IX – Disclaimer

THIS INFORMATION IS FURNISHED WITHOUT WARRANTY, EXPRESSED OR IMPLIED, EXCEPT THAT IT IS ACCURATE TO THE BEST OF OUR KNOWLEDGE.  THE DATA ON THIS SHEET RELATES ONLY TO THE SPECIFIC MATERIAL DESIGNATED HEREIN.  AIR-TECH COATINGS INC. ASSUMES NO LEGAL RESPONSIBILITY FOR USE OR RELIANCE UPON THIS DATA.