M. S. D. S. Material Safety Data Sheet

RESURFACER CURE PART B

SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NAME         : RESURFACER CURE PART B

PRODUCT CLASS        : EPOXY FILLER COMPOUND

HEALTH               : DANGER/CORROSIVE

Air-Tech Coatings, Inc.

1 Paradise Park Rd.

Jacksonville, AR  72076

(800)325-1650

EMERGENCY:800-424-9300 (ChemTrec)

24 Hours Emergency Hotline

PREPARE DATE: 10/10/10

PREVIOUS REVISION DATE: 02/10/05

SECTION 2 - COMPOSITION/INFORMATION ON INGREDIENTS

WT/WT %

ITEM  -------- CHEMICAL NAME ---------------  CAS # --------- LESS THAN -----------------------------------------------------------------------

01   		SILICA (QUARTZ**)*           14808-60-7          50.0 %

Fed OSHA: N  NTP: Y  ACGIH: N  IARC: 2A

* Cancer or cancer suspect agent.

02   		POLYAMINE ADDUCT             Mixture             30.0 %

03   		AROMATIC HYDROCARBON RESIN  25155-81-1           10.0 %

(Trace contaminant toluene# 0.6%, formaldehyde** 0.0001%)

04   	    TRIMETHYL HEXAMETHYLENEDIAMINE  3236-53-1            10.0 %

05   		RHEOLOGY ADDITIVE           8001-78-3             5.0 %

EXPOSURE LIMITS

----- ACGIH ----   ----- OSHA -----       VP          TOXICITY

TLV-TWA  TLV-TWA   PEL-TWA  PEL-TWA      mmHg     LD50     LC50

ITEM    ppm     Mg/M3      ppm     Mg/M3       @68F     g/kg     ppm

-----------------------------------------------------------------------

01     dna      0.05      dna      0.1        N.A.     dna      dna

02     dna      dna       dna      dna        N.A.     dna      dna

03     dna      dna       dna      dna        N.A.     dna      dna

04     dna      dna       dna      dna        N.A.     dna      dna

05     dna      3.0       dna      5.0        N.A.     dna      dna

REGULATORY:  **CALIF.TITLE 26:22-12000 (PROP 65). WARNING: This product contains a chemical known to the State of California to cause cancer. #CALIF.TITLE 26:22-12000 (PROP 65). WARNING: This product contains a chemical known to the State of California to cause birth defects or other reproductive harm.  All ingredients are on TSCA inventory or are exempt. Toxic chemicals marked (SARA, CERCLA, HAPs) are subject to reporting requirements of SARA (40CFR 355 and 372), CERCLA (40CFR 302), or HAPs (40CFR 63).

(S)=Skin; LD50=Dermal.rabbit; LC50=Inhalation,rat; dna=data not available; na=not applicable

SECTION 3 - HAZARDS IDENTIFICATION

EXPOSURE EFFECTS:  Vapor or spray mist or spattered material can be

harmful. Irritating to eyes, skin, and if inhaled; to nose and throat.

Excessive or prolonged inhalation can cause headache, nausea or                                                                                dizziness.

OVER-EXPOSURE (prolonged or repeated use):  CAN AGGRAVATE OR  ACCENTUATE ANY OF THESE EFFECTS.

SKIN:  Irritant.  Severe burns.  Sensitization or allergic reaction, such as rash or hives.  Can be absorbed through skin.

INHALATION:  Severe irritant.  Lung injury.  Repeated exposure to silica dust can cause silicosis.  Crystalline silica may cause cancer. Risk of cancer depends on duration and level of exposure to dust from sanding surfaces or spray mist.  High vapor concentrations may cause kidney and/or liver damage.

EYES:  Severe irritant.  Corneal injury.  Irreversible burns and damage.

INGESTION:  Can be fatal if swallowed.  Aspiration into lungs can damage lungs and cause chemical pneumonia.  Can cause burns.

TARGET ORGANS:  Kidneys.  Liver.  Blood.  Lungs.  Skin.  Eyes.  Stomach. Reproductive organs.

MEDICAL CONDITIONS AGGRAVATED:  Respiratory.  Lungs.

PRIMARY ROUTE(S) OF ENTRY: SKIN CONTACT  INHALATION  INGESTION

EYE CONTACT

SECTION 4 - FIRST AID MEASURES

FIRST AID PROCEDURES:

INHALATION: Remove to fresh air. Restore normal breathing. Treat symptomatically. See physician.

SKIN: Wash thoroughly with soap and water. Remove contaminated clothing. Consult physician if irritation persists.

EYES: Flush immediately with plenty of water for at least 15 minutes and get medical attention.

INGESTION: Drink 1 or 2 glasses of water to dilute. Never give anything by mouth to an unconscious person. Do not induce vomiting. Consult physician or poison control center IMMEDIATELY. Treat symptomatically.  EYES: After flushing eyes for 15 minutes, get IMMEDIATE medical attention from an ophthalmologist.

SECTION 5 - FIRE FIGHTING MEASURES

FLASH POINT: 200 F (SETA)                   LOWER EXPLOSIVE LIMIT: N.A.

UPPER EXPLOSIVE LIMIT: N.A.

FLAMMABILITY -  OSHA: COMBUSTIBLE - CLASS IIIB

DOT: NOT REGULATED

EXTINGUISHING MEDIA:  FOAM  CO2  DRY CHEMICAL

LOWEST FLASHING SOLVENT:

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Closed containers may explode when

exposed to extreme heat and pressure buildup.

FIREFIGHTING PROCEDURES:  Wear full protective equipment,self-contained

breathing apparatus. Water may be used to cool closed containers to    prevent pressure build-up or explosion when exposed to extreme heat.

SECTION 6 - ACCIDENTAL RELEASE MEASURES

SPILL, LEAKS:  Ventilate area. Use inert, absorbent cleanup materials. (DO NOT use sawdust.) Place in separate container.  Keep out of sewers and waterways. If entry is threatened or occurs, notify local authorities.

SECTION 7 - HANDLING AND STORAGE

HANDLING AND STORAGE:  Keep container closed, upright when not in use.

Store in cool, dry, well-ventilated area. Avoid prolonged storage

temperatures above 100F.  Use caution when pouring. Avoid breathing sanding dust.  Do not weld or flame cut on empty container.

SECTION 8 - EXPOSURE CONTROLS/PERSONAL PROTECTION

VENTILATION:  Implement administrative and engineering controls to reduce exposure. Provide sufficient ventilation in volume and pattern to keep air contaminant concentrations below the TLV limits. Remove welding or flame cutting decomposition products; follow current,ANSI Z49.1,"Safety in Welding and Cutting". Refer to 29 CFR parts 1910 and 1915, for coating operations; part 1910.146, Confined Spaces.

RESPIRATORY PROTECTION:  Wear NIOSH/MSHA certified respirator designed to remove a combination of particulates (dust or spray mist) and vapor. When brushing, rolling or spreading; select the appropriate respiratory

protection for the conditions. For specific conditions, refer to current "NIOSH Pocket Guide to Chemical Hazards". In confined or restricted ventilation areas use air-line respirators or hoods. Refer to 29 CFR, OSHA parts 1910.134 and 1915 for coating operations;part 1910.146 Confined Spaces;ANSI Z88.2,Practices for Respiratory Protection; 42 CFR, part 84 Particulate Respirators.

PROTECTIVE CLOTHING AND EQUIPMENT:  Dependent upon application method, wear resistant coveralls, gloves and shoe coverings to prevent skin contact. Wear solvent resistant glasses with splashguards or face shield to protect eyes from splash, spatter and/or spray mist. Consult 29 CFR 1910.132, 133, 136, 138; ANSI Z87.1, Z41.

HYGIENIC PRACTICES:  Wash thoroughly after handling and before eating,

smoking or using toilet. Launder contaminated clothing before use.  Destroy contaminated leather and absorbent shoes, which cannot be decontaminated, to prevent reuse.

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

BOILING RANGE     : N.A.         VAPOR DENSITY   : Is heavier than air

ODOR              : NA           WEIGHT PER GAL  : 11.5077

APPEARANCE        : LIQUID       EVAPORATION RATE: Is slower than Butyl

SOLUBILITY IN H2O : NO                             Acetate

MIXED VOC, G/L    : 0            PHOTOCHEMICALLY REACTIVE: No

VOLATILE VOLUME % : 0.00

SECTION 10 - STABILITY AND REACTIVITY

CONDITIONS TO AVOID:  Heat, open flame, arc or sparks.  Epoxies under

uncontrolled conditions.

INCOMPATIBILITY:  Strong oxidizers, acids and alkalies.

HAZARDOUS DECOMPOSITION PRODUCTS:  (BY FIRE, BURNING OR WELDING); CO, CO2. Acrylic monomer fumes. Ammonia

HAZARDOUS POLYMERIZATION: Will not occur under normal conditions.

STABILITY: This product is stable under normal storage conditions.

SECTION 11 - TOXICOLOGICAL PROPERTIES

TOXICOLOGICAL PROPERTIES:  See Section 2.  Crystalline silica (respirable size, 10 microns or less). IARC Monograph on the Evaluation of Carcinogenic Risk of Chemicals to Humans (Vol 68.1997) concludes that there is sufficient evidence of carcinogenicity to experimental animals and limited evidence of carcinogenicity to humans. IARC Class 1

SECTION 12 - ECOLOGICAL INFORMATION

ECOLOGICAL INFORMATION:  No Information.

SECTION 13 - DISPOSAL CONSIDERATIONS

EPA Waste No.: None

DISPOSAL METHOD:  Place in separate, appropriate, closed container in

accordance with all applicable local, State, and Federal regulations. This material has NOT been tested by Toxicity Characteristic Leaching Procedure (TCLP).

SECTION 14 - TRANSPORTATION INFORMATION

DOT PROPER SHIPPING NAME: Paint Related Material

DOT HAZARD CLASS: 8                        HAZARD SUBCLASS: NA

DOT UN/NA NUMBER: 3066             IMO: NA         PACKING GROUP: III

SECTION 15 - REGULATORY INFORMATION

U.S. FEDERAL REGULATIONS: AS FOLLOWS -

OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR

1910.1200)

NEW JERSEY RIGHT-TO-KNOW:

The following materials are non-hazardous, but are among the top five

components in this product:

----------- CHEMICAL NAME -----------    CAS NUMBER

No non-hazardous materials are among the top five ingredients.

PENNSYLVANIA RIGHT-TO-KNOW:

The following non-hazardous ingredients are present in the product at

greater than 3%:

----------- CHEMICAL NAME ----------- CAS NUMBER

No non-hazardous ingredients are present at greater than 3%.

INTERNATIONAL REGULATIONS: AS FOLLOWS -

CANADIAN WHMIS: This MSDS has been prepared in compliance with  Controlled Product Regulations except for use of the 16 headings.

CANADIAN WHMIS CLASS: No information available.

SECTION 16 - OTHER INFORMATION

NOTICE:  Removal of old lead paint by sanding, scraping or other means may generate dust or fumes that contain lead. Exposure to lead dust or fumes may cause adverse health effects, especially in children or pregnant women. Controlling exposure to lead or other hazardous substances requires the use of proper protective equipment, such as a properly fitted respirator (NIOSH approved) and proper containment and cleanup. For additional information, contact the USEPA/Lead Information Hotline at 1-800-424-LEAD.

The information contained in this MSDS is based on data from sources considered to be reliable but Air Tech Coatings, Inc. does not guarantee the accuracy or completeness thereof.  Air Tech Coatings, Inc. urges each customer or recipient of this MSDS to study it carefully to become aware of and understand the hazards associated with this product.  The reader should consider consulting reference works or individuals who are experts in ventilation, toxicology or fire prevention as necessary or appropriate to use and understand the data in this MSDS.