COMPANY IDENTITY:  AIR TECH COATINGS, INC.		 Preparation Date: 10/10/10  Supersedes Date:  07/01/09

PRODUCT  IDENTITY:  HARD DECK RESIN							PAGE     1

MATERIAL SAFETY DATA SHEET

THIS MSDS COMPLIES WITH 29 CFR 1910.  1200  (HAZARD COMMUNICATION STANDARD).

IMPORTANT:  Read this MSDS before handling and disposing of this product.  Pass this information on to employees, customers, & users of this product.

SECTION 1; CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

PRODUCT IDENTITY:  HARD DECK RESIN

NAME:  AIR TECH COATINGS, INC.   			     	ADDRESS:  #1 PARADISE PARK RD.

CITY:  JACKSONVILLE, AR 72076			     	PHONE:  1-501-985-1484

INFOTRAC PHONE:  1-800-535-5053

SECTION 2;  INGREDIENT & REGULATORY INFORMATION

All components of this product are on the TSCA list.

SARA Title III Section 313 Supplier Notification

This product contains the <*> toxic chemicals subject to the reporting requirements of Section 313 of the Emergency Planning & Community Right-to-Know Act of 1986 & of 40 CFR 372.  This information must be included in all MSDS’s that are copied and distributed for this material.

SARA TITLE III INGREDIENTS			CAS #		WT. %	   (REG. SECTION)  RQ(LBS)

ETHYL ACETATE			141-78-6	NA	    (311, 312)	       NONE

METHYL n-PROPYL KETONE		107-87-9	NA	    (311, 312)	       NONE

*TOLUENE				108-88-3		    (311, 312, 313, RCRA)  1000

ETHYL-3-ETHOXYPROPIONAT		763-69-9	NA	    (311, 312)	       NONE

*METHYL ISOBUTYL KETONE		108-10-1		    (311, 312, 313, RCRA)  5000

2-Methoxypropyl Acetate (PMA)		108-65-6		    (311, 312, 313, RCRA)

XYLENE				1330-20-7		    (311, 312, 313, RCRA)

POLYESTER RESIN, FILM FORMERS, ADDITIVES            58

SARA SECTION 311/312 HAZARDS:  Acute Health, Fire

MATERIAL				CAS #	       TWA +  (OSHA)	TLV (ACGIH)

ETHYL ACETATE			141-78-6	400 PPM	400 PPM

METHYL N-PROPYL KETONE		107-87-9	200 PPM	200 PPM

TOLUENE				108-88-3	100 PPM	100 PPM

ETHYL-3-ETHOXYPROPIONAT		763-69-9	  50 PPM	  50 PPM

METHYL ISOBUTYL KETONE		108-10-1	  50 PPM	  50 PPM

2-Methoxypropyl Acetate (PMA)		108-65-6	100 PPM	Not est

XYLENE				1330-20-7	100 PPM	100 PPM

MATERIAL 				CAS #		CEILING	STEL (OSHA/ACGIH)

TOLUENE				108-88-3	300 PPM	150 PPM

METHYL ISOBUTYL KETONE		108-10-1	None Known	75 PPM

Xylene					1330-20-7	100 ppm		150 PPM

MATERIAL				CAS #	          LOWEST KNOWN LETHAL DOSE DATA

LOWEST KNOWN LD50 (ORAL)

METHYL ISOBUTYL KETONE		108-10-1          2080.0 MG/KG (RATS)

LOWEST KNOWN LC50 (VAPORS)

ETHYL ACETATE			141-78-6          1600 PPM (RATS)

LOWEST KNOWN LD50 (SKIN)

TOLUENE				108-88-3           4000.0 MG/KG (RABBITS)

HAZARDS:  HEALTH (NFPA) 2     HEALTH (HMIS):  1     FLAMMABILITY:  3   REACTIVITY:  0

SECTION 2:  INGREDIENT & REGULATORY INFORMATION

CALIFORNIA PROPOSITION 65:  This product contains the following chemical known to the state of California to cause reproductive toxicity:

Toluene

DOT SHIPPING NAME:  PAINT RELATED MATERIAL, 3, (UN1263), PG-II

DRUM LABEL:  3 (FLAMMABLE LIQUID)

SECTION 3;  HAZARDS IDENTIFICATION

THRESHOLD LIMIT VALUE:  140 PPM (EVAPORATED BLEND)

CONTAINS:  ETHYL ACETATE, METHYL n-PROPYL KETONE, TOLUENE, ETHYL-3-ETHOXYPROPIONAT, METHYL ISOBUTYL KETONE, 2-METHOXYPROPYL ACETATE (PMA), XYLENE

WARNING!

FLAMMABLE!!

-ACUTE HAZARDS

EYE & SKIN CONTACT:

Primary irritation to skin, defatting, dermatitis.  Adsorption through skin increases exposure.

Primary irritation to eyes, redness, tearing, blurred vision.

Liquid can cause eye irritation.  Wash thoroughly after handling.

INHALATION:

Anesthetic.  Irritates respiratory tract.  Acute overexposure can cause serious nervous system depression.  Vapor harmful.

Breathing vapor can cause irritation.

Acute overexposure can cause damage to kidneys, blood, nerves, liver, & lungs.

SWALLOWING:

Harmful or fatal if swallowed.

Swallowing can cause abdominal irritation, nausea, vomiting, & diarrhea.

SUBCHRONIC HAZARDS/ CONDITIONS AGGREVATED

SUBCHRONIC HAZARDS

Absorption through skin may be harmful.  Studies with laboratory animals indicate this product can cause damage to fetus.

Chronic overexposure can cause damage to kidneys, blood, nerves, liver & lungs.

Persons with asthma, chronic respiratory problems, severe heart, skin, liver or kidney problems should avoid use.

CHRONIC HAZARDS

This product has no carcinogens listed by 1ARC, NIP, NIOSH, OSHA, or ACGIH, as of this date, greater or equal to 0.1%.

This product may contain less than 100 ppm of Benzene.

Not considered in such low concentrations.

SECTION 4;  FIRST AID MEASURES PROCEDURES

EYE CONTACT:

For eyes, immediately flush with plenty of water for 15 minutes & CALL A PHYSICIAN.

SKIN CONTACT:

In case of contact with skin, immediately remove contaminated clothing.  Wash thoroughly with soap & water.  Wash contaminated clothing before reuse.     (Discard contaminated shoes.)

INHALATION:

After high vapor pressure, remove to fresh air,  If breathing is difficult, give oxygen.  If breathing has stopped give artificial respiration.  CALL        A PHYSICIAN IMMEDIATELY!!

SWALLOWING:

If swallowed, CALL A PHYSICIAN IMMEDIATELY!  Never give anything by mouth to an unconscious person.

SECTION 5;  FIRE FIGHTING MEASURES

LOWER FLAMMABLE LIMIT IN AIR (% BY VOLUME):  1.8

FLASH POINT (TEST METHOD):  70 F (PMCC)

FLAMMABILITY CLASSIFICATION:  CLASS I B

EXTINGUISHING MEDIA

NFPA Class B extinguishers (Carbon Dioxide or foam) for Class I B liquid fires.

SPECIAL FIRE FIGHTING PROCEDURES

Water spray may be ineffective on fire but can protect fire-fighters & cool closed containers.  Use fog nozzles if water is used.

Do not enter confined fire-space without full bunker gear.  (Helmet with face shield, bunker coats, gloves & rubber boots).

Use NIOSH approved positive-pressure self-contained breathing apparatus.

UNUSUAL EXPLOSION AND FIRE PROCEDURES

FLAMMABLE!!

Keep container tightly closed.

Isolate from oxidizers, heat, sparks, electric equipment & open flame.

Closed containers may explode if exposed to extreme heat.

Applying to hot surfaces requires special precautions.

Empty container very hazardous!  Continue all label precautions!!

SECTION 6;  ACCIDENTAL RELEASE  MEASURES

SPILL OR LEAK PROCEDURES:

Stop spill at source.  Dike area & contain.  Clean up area with absorbent materials.  Mop up & dispose of.  Persons without proper protection	        should be kept from area until cleaned up.

WASTE DISPOSAL METHOD:

Recycle or dispose of observing local, state & Federal health, safety & pollution laws.  If questions exist, contact the appropriate agencies.

OTHER PRECAUTIONS:

None

SECTION 7;  HANDLING AND STORAGE

HANDLING

Isolate from oxidizers, heat, sparks, electric equipment & open flame.  Use only with adequate ventilation.  Avoid breathing of vapor or spray mist.

Avoid contact with skin & eyes.

Near OSHA Standard goggles or face sheild.  Consult Safety Equipment supplier.

Wear gloves, apron & footwear impervious to this material.  Wash clothing before reuse.

Avoid free fall of liquid.  Ground containers when transferring.  Do not flame, cut, saw, drill, braze, or weld.  Emoty container very hazardous!!     Continue all label precautions!

STORAGE

Do not store above 49 C/120F.  Store large amounts in structures made for OSHA Class I B liquids.  Keep container tightly closed & upright when not in       use to prevent leakage.

SECTION 8;  EXPOSURE CONTROLS/ PERSONAL PROTECTION:

EXPOSURE CONTROLS:

Ventilate to keep vapors of this material below 70 ppm.  If over TLV, in accordance with 29 CFR 1910.134, use NIOSH approved positive-pressure self       contained breathing apparatus.  Consult Safety Equipment Supplier.  Use explosion-proof equipment.

VENTILATION:

LOCAL EXHAUST			:  NECESSARY

MECHANICAL (GENERAL)		:  ACCEPTABLE

SPECIAL				:  NONE

OTHER					:  NONE

PERSONAL PROTECTIONS:

Wear OSHA Standard goggles or face shield.  Consult Safety Equipment Supplier.

Wear gloves, apron & footwear impervious to this material.  Wash clothing before reuse.

SECTION 9;  PHYSICAL DATA

APPEARANCE:  						LIQUID, WATER WHITE

ODOR:								KETONE

BOILING RANGE:						76  103  166  C  /  170  218  331  F

GRAVITY @ 60F:

API:							     32.2

SPECIFIC GRAVITY (WATER=1)				     1.036

POUNDS/GALLON  :					      8.63

VOC’S (VAPOR PRESSURE   >0.44 LBS/SQ  IN) (LBS/GAL)	                     3.58

TOTAL NONVOLATILE ORGANIC COMPOUNDS (TVOC)  (G/L):	                    587.7

NONEXPEMPT VOLATILE ORGANIC COMPOUNDS (CVOC) (G/L):                587.7

NONEXEMPT VOLATILE ORGANIC COMPOUNDS (CVOC)  (VOL %)              47.4

VAPOR PRESSURE  (MM OF HG) @20C					       43.0

VAPOR DENSITY (AIR=1)						        3.4

WATER ABSORPTION  :						   APPRECIABLE

% VOLATILE BY VOLUME  :						      47.4

SECTION 10;  REACTIVITY DATA

STABILITY

Stable

CONDITIONS TO AVOID

Isolate from oxidizers, heat, sparks, electric equipment & open flame.

MATERIALS TO AVOID

Isolate from strong oxidizers such as permanganates, chromates and peroxides.

HAZARDOUS DECOMPOSITION PRODUCTS

Carbon Monoxide, Carbon Dioxide from burning.

HAZARDOUS POLYMERIZATION

Will not occur.

NOTICE

Supplier disclaims all expressed or implied warranties of merchantability or fitness for a specific use, with respect to the product or the information provided herein, except for confirmation to contracted specifications.

All information appearing herein is based upon data obtained from manufacturers and/or recognized technical sources.  While this information is believed to be accurate, we make no representations as to its accuracy or sufficiency.

Conditions of use are beyond our control, and therefore users are responsible for verifying the data under their own operating conditions to determine whether the product is suitable for their particular purposes and they assume all risks of their use, handling and disposal of the product.  Users also assume all risks in regards to the publication or use of, or reliance upon information contained herein.

This information relates only to the product designated herein, and does not relate to its use in combination with any other materials or process.