Material Safety Data Sheet

Company Identity:  Air-Tech Coatings, Inc.

Product Identity:  RT 4400

Reducer

Date of Preparation:  10/10/10

This MSDS complies with 29 CFR 1910.  1200 (Hazard Communication Standard).  Important:  Read this MSDS before handling and disposing of this product.  Pass this information on to employees, customers, and users of this product.

# Section 1 – Chemical Product and Company Identity

Product Identity:		RT 4400 Reducer

Name:			Air-Tech Coatings, Inc.

Address:			#1 Paradise Park Road

City:			Jacksonville, AR  72076

Phone:			1-501-985-1484

Infotrac Phone:		1-800-535-5053

# Section 2 – Ingredient & Regulatory Information

All components of this product are on the TSCA list.

SARA Title III Section 313 Supplier Notification

This product contains the <*> toxic chemicals subject to the reporting requirements of Section 313 of the Emergency Planning & Community Right-to-Know Act of 1986 and of 40 CFR 372.  This information must be included in all MSDS’s that are copied and distributed for this material.

SARA Title III Ingredients		CAS #			WT. %		REG. Section		RQ (LBS)

Methyl Amyl Ketone			110-43-0		NA		(25)			None

Xylene					1330-20-7		NA		(311, 312) 20		None

SARA Section 311/312 Hazards:  Acute Health, Fire

Material					CAS #			TWA + (OSHA)		TLV

(ACGIH)

Methyl Amyl Ketone			110-43-0		100 ppm			50 ppm

Xylene					1330-20-7		100 ppm			100 ppm

Material					CAS #			Ceiling			STEL

(OSHA/ACGIH)

Methyl Amyl Ketone			110-43-0		100 ppm			50 ppm

Xylene					1330-20-7		None Known		150 ppm

California Proposition 65:  This product contains the following chemical known to the State of California to cause reproductive toxicity:

N/A

DOT Shipping Name:  Paint Related Material, 3, (UN1263), PG-II

Drum Label:  3 (Flammable Liquid)

Page 1

# Section 3 – Hazards Identification

Threshold Limit Value:  140 ppm (Evaporated Blend)

Contains:  Methyl Amyl Ketone;  Xylene

# Warning!

# Flammable!!

# Acute Hazards

Eye & Skin Contact:  Primary irritation to skin, defatting, dermatitis.  Absorption through skin increases exposure.  Primary irritation to eyes, redness, tearing, blurred vision.  Liquid can cause eye irritation.  Wash thoroughly after handling.

Inhalation:  Anesthetic.  Irritates respiratory tract.  Acute overexposure can cause serious nervous system depression.  Vapor harmful.  Breathing vapor can cause irritation.  Acute overexposure can cause damage to kidneys, blood, nerves, liver & lungs.

Swallowing:  Harmful or fatal if swallowed.  Swallowing can cause abdominal irritation, nausea, vomiting and diarrhea.

# Subchronic Hazards/Conditions Aggravated

## Subchronic Hazards

Absorbtion through skin may be harmful.  Studies with laboratory animals indicate this product can cause damage to fetus.  Chronic overexposure can cause damage to kidneys, blood, nerves, liver and lungs.  Persons with asthma, chronic respiratory problems, severe heart, skin, liver or kidney problems should avoid use.

# Section 4 – Accidental Release Measures

Spill or Leak Procedures:  Stop spill at source.  Dike area and contain.  Clean up remainder with absorbent materials.  Mop up and dispose of.  Persons without proper protection should be kept from area until cleaned up.

Waste Disposal Method:  Recycle or dispose of observing local, state and federal health, safety and pollution laws.  If questions exist, contact the appropriate agencies.

Other Precautions:  None

# Section 5 – Handling and Storage

Handling:  Isolate from oxidizers, heat, sparks, electric equipment and open flame.  Use only with adequate ventilation.  Avoid breathing of vapor or spray mist.  Avoid contact with skin and eyes.  Wear OSHA Standard goggles or face shield.  Consult Safety Equipment supplier.  Wear gloves, apron and footwear impervious to this material.  Wash clothing before reuse.  Avoid free fall of liquid.  Ground containers when transferring.  Do not flame, cut, saw, drill, braze, or weld.  Empty container very hazardous!!  Continue label precautions!

Storage:  Do not store above 49 C/120F.  Store large amounts in structures made for OSHA Class I B liquids.  Keep container tightly closed and upright when not in use to prevent leakage.

# Section 6 – Exposure Controls/Personal Protection

Exposure Controls:  Ventilate to keep vapors of this material below 70 ppm.  If over TLV, in accordance with 29 CFR 1910.134, use NIOSH approved positive-pressure self contained breathing apparatus.  Consult Safety Equipment Supplier.  Use explosion-proof equipment.

Ventilation:

Local Exhaust:		Necessary

Mechanical (General):	Acceptable

Special:			None								Page 2

Other Personal Protections:  Wear OSHA Standard goggles or face shield.  Consult Safety Equipment Supplier.  Wear gloves, apron and footwear impervious to this material.  Wash clothing before reuse.

# Section 7 – Physical Data

Appearance:	Liquid, water white

Odor:		Ketone

Boiling Range:	76 103 166 C / 170 218 331 F

Gravity @ 60F:	API:  32.2

Specific Gravity (Water=1)			0.864

Pounds/Gallons:					7.200

VOC’s (Vapor Pressure >0.44 lbs/sq in) (lbs/gal)	6.414

Total Nonvolatile Organic Compounds (TVOC) (G/L):  864.4

Nonexempt Volatile Organic Compounds (CVOC) (VOL%)	100.0

Vapor Pressure (MM of HG)@20C			43.0

Vapor Density (AIR=1)				3.4

Water Absorption:				Appreciable

% Volatile by Volume:				100.0

Refractive Index:					1.409

# Section 8 – Reactivity Data

Stability:  Stable

Conditions to Avoid:  Isolate from oxidizers, heat, sparks, electric equipment and open flame.

Materials to Avoid:  Isolate from strong oxidizers such as permanganates, chromates and peroxides.

Hazardous Decomposition Products:  Carbon Monoxide, Carbon Dioxide from burning.

Hazardous Polymerization:  Will not occur.

# Notice

Supplier disclaims all expressed or implied warranties of merchantability or fitness for a specific use, with respect to the product or the information provided herein, except for conformation to contracted specifications.  All information appearing herein is based upon data obtained from manufacturers and/or recognized technical sources.  While this information is believed to be accurate, we make no representations as to its accuracy or suffiency.

Conditions of use are beyond our control, and therefore users are responsible for verifying the data under their own operating conditions to determine whether the product is suitable for their particular purposes and they assume all risk of their use, handling and disposal of the product.  Users also assume all risks in regards to the publication or use of, or reliance upon, information contained herein.

This information relates only to the product designated herein, and does not relate to its use in combination with any other materials or process.  This product has no carcinogens listed by IARC, NTP, NIOSH, OSHA, or ACGIH, as of this date, greater than or equal to 0.1%  This product may contain less than 100ppm of Benzene.  Not considered hazardous in such low concentrations.

Lowest known Lethal Dose Data

Hazards:  Health   (NFPA) 2	Health (HMIS):  1	Flammability:  3		Reactivity:  0

Page 3